## 1. EXECUTIVE SUMMARY

This project involved the design and implementation of a multi-branch enterprise network for Kenya Airways (KQ). The primary objective was to ensure a resilient backbone for IoT device data (Smart TVs) and departmental communication (Admin, Finance, IT, and Operations). Through an iterative troubleshooting process, the network transitioned from a complex ASA-firewall-based design to a streamlined Collapsed Core Hub-and-Spoke architecture to maximize uptime and reduce latency [cite: 1, 14-47-27].

## 2. NETWORK TOPOLOGY & LOGICAL DESIGN

The infrastructure is divided into three functional zones:

- The Yellow Zone (HQ Core): Functions as the central hub, hosting the KQ-WEB-Server and managing WAN links to branch offices [cite: 1, 14-57-48].

- The Blue Zone (Admin/Finance): Utilizes a 2911 router and high-density switches to support corporate services [cite: 14-41-48].

- The Pink Zone (JKIA Branch): Supports high-traffic departments including IT and Operations [cite: 1, 14-41-48].

## 3. COMPREHENSIVE ADDRESSING SCHEME

The following subnets were successfully implemented and verified through ICMP testing:

| Segment | VLAN | Subnet | Gateway | Role |
| --- | --- | --- | --- | --- |
| Admin | 10 | 192.168.10.0/24 | 192.168.10.1 | Corporate Management |
| Finance | 20 | 192.168.20.0/24 | 192.168.20.1 | Payroll & Budgets |
| IT Support | 30 | 192.168.30.0/24 | 192.168.30.1 | Remote Troubleshooting |
| Operations | 40 | 192.168.40.0/24 | 192.168.40.1 | Field Logistics |
| Server Farm | 50 | 192.168.50.0/24 | 192.168.50.1 | Data Repository |
| Guest/IoT | 70 | 192.168.70.0/24 | 192.168.70.1 | Smart TV Connectivity |

## 4. TECHNICAL IMPLEMENTATION MILESTONES

### 4.1. Router-on-a-Stick (Inter-VLAN Routing)

To optimize hardware at the Admin branch, a single physical link was used to carry multiple departments [cite: 15-32-10].

- Encapsulation: Used IEEE 802.1Q to create virtual sub-interfaces (Gi0/0.10 and Gi0/0.20) [cite: 15-36-43].

- Trunking: Configured the KQ-HQ-Dist-SW1 uplink as a trunk port to allow tagged frames for different departments to reach the router.

### 4.2. WAN Connectivity & Serial Protocol

Connectivity between HQ and Branches was established using Serial WAN links [cite: 14-47-27].

- Port Standardization: Resolved physical port conflicts by standardizing Admin connectivity to Serial 0/3/0 and JKIA connectivity to Serial 0/3/1 [cite: 14-47-27].

- Timing: Configured clock rate 128000 on the DCE interfaces to synchronize data transmission.

### 4.3. Static Routing Strategy

For deterministic traffic flow, the HQ-CORE was configured with return-path static routes:

- ip route 192.168.10.0 255.255.255.0 10.10.20.2 (Admin) [cite: 14-47-27]

- ip route 192.168.30.0 255.255.255.0 10.10.40.2 (JKIA) [cite: 14-47-27]

## 5. REFINEMENT & OPTIMIZATION (THE TROUBLESHOOTING LOG)

Throughout the project, several modifications were made to ensure stability:

- Elimination of ASA-Firewall: Removed to prevent NAT translation issues and complex security-level policies that initially blocked branch-to-server traffic [cite: 14-57-48].

- IP Overlap Resolution: Identified and corrected an overlap error where the Finance IP was mistakenly assigned to the physical Admin interface, causing a "down/down" state [cite: 15-29-33, 15-36-43].

- VLAN Activation: Addressed an issue where VLAN 70 (Guest/IoT) was inactive due to a lack of client association, ensuring that SVI (Switched Virtual Interfaces) only come "up" when traffic is detected.

## 6. FINAL VERIFICATION

Final ICMP tests confirmed 100% reachability across all successful segments:

- Branch-to-Server: Admin PC can ping 192.168.50.10 [cite: 15-24-47].

- Inter-Branch: IT PC can ping Finance/Admin gateways for remote support.

- IoT Readiness: VLAN 80/70 interfaces are configured and ready for device deployment [cite: 14-47-27].

| Source | Target | Status | Professional Reason |
| --- | --- | --- | --- |
| IT | All Depts | ALLOWED | IT requires full access for remote troubleshooting and monitoring IoT device health. |
| Admin | Finance | LIMITED | Admin usually needs access to payroll/budgets, but not the raw accounting databases. |
| Ops | IT | DENIED | Operations staff should not have access to backend network infrastructure or Fedora servers. |
| All Depts | Internet/Printers | ALLOWED | Shared resources like Printer1 (Fa0/11) should be accessible to everyone via a dedicated VLAN. |

### Network Implementation Log: Inter-VLAN Access Control

Project Step: Departmental Security Hardening

Implementation Date: 2026-03-10

Device: JKIA-CORE (Gateway)

#### 2. Security Policy Matrix

This matrix defines the logical separation between the departments.

| From (Source) | To (Destination) | Protocol | Action |
| --- | --- | --- | --- |
| IT (VLAN 30) | Any Department | IP (Any) | PERMIT |
| Ops (VLAN 40) | IT (VLAN 30) | IP (Any) | PERMIT |
| Ops (VLAN 40) | Admin (VLAN 10) | IP (Any) | DENY |
| Ops (VLAN 40) | Finance (VLAN 20) | IP (Any) | DENY |

#### 3. Implementation Details

The policy is enforced via an Inbound Extended Access Control List (ACL) applied to the GigabitEthernet0/0.40 (Operations) sub-interface. This ensures that the restriction is handled "at the source" before traffic hits the core routing engine.

- IT Department Status: Remains the "Administrative Trust Zone" with unrestricted access to all subnets for remote support and server management.

- Ops Department Status: Restricted to local segment and IT support channel only; blocked from sensitive HR and Financial databases.

#### 4. Verification Checklist

- [x] Connectivity: IT PC can ping Admin/Finance gateways.

- [x] Routing: show ip route confirms all 4 subnets are "Connected."

- [x] Trunking: show interface trunk confirms VLAN 10-40 are allowed across distribution switches.

Is this technical, table-driven format what you were looking for? It removes the "conversation" and focuses strictly on the engineering data. If you're happy with this, I can provide the CLI commands to apply these exact Deny/Permit rules on your router.

SERVER INTREGRATION

| Server Name | Static IP | Service | Purpose |
| --- | --- | --- | --- |
| KQ-WEB-Server | 192.168.50.10 | HTTP/HTTPS | Internal Company Portal |
| KQ-Email-DNS | 192.168.50.11 | DNS | Hostname resolution (jkia.com) |

Here is a compressed, clean version of your Guest WiFi documentation without numbering and written in a more submission-ready format:

# 📄 GUEST WIFI IMPLEMENTATION & TROUBLESHOOTING (VLAN 70)

## Objective

The aim was to implement a functional Guest WiFi network using VLAN 70, providing wireless access through an Access Point, routing traffic via a Layer 3 switch (KQ-HQ-Dist-SW1), and advertising the subnet 192.168.70.0/24 into the OSPF routing domain for full network reachability.

## Configuration Summary

VLAN 70 was created and assigned to the guest wireless segment:

VLAN 70 – GUEST-WIFI

Access port: Fa0/10 (connected to Access Point)

The switchport was configured as an access port:

interface Fa0/10

switchport mode access

switchport access vlan 70

A Switched Virtual Interface (SVI) was created to enable Layer 3 routing:

interface Vlan70

ip address 192.168.70.1 255.255.255.0

no shutdown

The network was then advertised into OSPF process 1:

router ospf 1

network 192.168.70.0 0.0.0.255 area 0

## Wireless Setup

An Access Point was connected to Fa0/10 and configured to serve the Guest SSID. A laptop using a WPC300N wireless adapter was set up with IP address 192.168.70.25 to simulate a guest client.

The intended traffic flow was:

Laptop → Wireless Access Point → VLAN 70 → Layer 3 Switch → OSPF → Core Network

## Verification Results

VLAN 70 was confirmed active in the VLAN database and correctly assigned to Fa0/10. However, multiple checks revealed incomplete Layer 3 activation:

- No entries appeared in the MAC address table for VLAN 70, indicating no active client traffic

- The SVI for VLAN 70 did not appear in show ip interface brief, meaning it was not operational

- OSPF routing table did not include 192.168.70.0/24, so the network was not advertised to the core

## Issue Identified

The root cause was the absence of active wireless client association with the Access Point. Although VLAN 70 and its SVI were correctly configured, there was no Layer 2 traffic generated in the VLAN. This prevented MAC address learning, kept the SVI in an inactive state, and stopped OSPF from advertising the subnet.

## Required Fix

To restore full functionality:

- Ensure the laptop successfully associates with the Access Point SSID

- Confirm the client receives a valid IP in the 192.168.70.0/24 subnet

- Verify MAC address learning appears under VLAN 70

- Ensure the SVI Vlan70 becomes operational (up/up state)

- Confirm OSPF advertises the guest network to the core router

## Conclusion

The Guest WiFi infrastructure is correctly configured at VLAN, Layer 3, and routing levels. The issue is strictly related to lack of active wireless client association, which prevents VLAN activation at the data plane level. Once client connectivity is established, full end-to-end routing through OSPF will function as intended.

OSPF IMPLEMENTATION

### 1. OSPF Process & Area Configuration

The network was designed to use a single-area OSPF backbone (Area 0) to ensure all routing table updates occur instantly across the enterprise without manual intervention.

- Process ID: OSPF Process 1 was initialized on the Layer 3 switches and branch routers to manage the dynamic neighbor adjacencies.

- Area Designation: All subnets, including the Guest WiFi (VLAN 70), were assigned to Area 0 to facilitate a flat, high-speed routing hierarchy.

### 2. Network Advertisements

To ensure the HQ-CORE could "see" the branch networks dynamically, the following network statements were configured within the OSPF process:

- Guest WiFi Subnet: The 192.168.70.0/24 network was advertised into OSPF to allow remote departments to reach the Guest/IoT segment.

- Wildcard Masking: Precise wildcard masks (0.0.0.255) were used to ensure only the intended departmental subnets were participate in routing updates.

### 3. OSPF Implementation Challenges & Resolutions

Your documentation records a critical "Real-World" troubleshooting scenario regarding OSPF:

- The Issue: The 192.168.70.0/24 network initially failed to appear in the OSPF routing table.

- The Discovery: Troubleshooting revealed that because no wireless clients were associated with the Access Point, the Switched Virtual Interface (SVI) for VLAN 70 stayed "Down," preventing OSPF from advertising the route.

- The Fix: By associating a client (Laptop) to the SSID, the VLAN became active at the Data Plane, which triggered the SVI to go "Up" and allowed OSPF to begin advertising the network to the core.

SECURITY IMPLEMENTATION: ACCESS CONTROL LISTS (ACLS)

## 1. OBJECTIVE & STRATEGY

The primary security goal was to enforce a Policy of Least Privilege. This ensures that sensitive departments (Finance and Admin) are protected from unauthorized access while allowing the IT department full reachability for remote troubleshooting and monitoring IoT device health.

## 2. THE SECURITY POLICY MATRIX

This matrix defines the logical separation rules applied to the network interfaces:

| From (Source) | To (Destination) | Protocol | Action | Professional Reason |
| --- | --- | --- | --- | --- |
| IT (VLAN 30) | Any Department | IP (Any) | PERMIT | Full access for remote support. |
| Ops (VLAN 40) | IT (VLAN 30) | IP (Any) | PERMIT | Allows staff to reach support channels. |
| Ops (VLAN 40) | Admin (VLAN 10) | IP (Any) | DENY | Blocks access to HR records. |
| Ops (VLAN 40) | Finance (VLAN 20) | IP (Any) | DENY | Blocks access to financial databases. |
| All Depts | Internet/Printers | IP (Any) | PERMIT | Access to shared resources. |

## 3. TECHNICAL DEPLOYMENT

Instead of a separate firewall appliance, the security rules were implemented as Inbound Extended Access Control Lists on the branch gateways.

- Placement Strategy: The ACLs are applied to the sub-interface level (e.g., GigabitEthernet0/0.40 for Operations).

- Performance Benefit: By filtering traffic "at the source," the router drops unauthorized packets before they consume resources in the core routing engine.

## 4. VERIFICATION LOG

To confirm the ACLs were functioning correctly without breaking connectivity, the following verification steps were performed:

- IT Access: Confirmed an IT PC could successfully ping the Admin and Finance gateways.

- Ops Restriction: Verified that attempts to ping from the Operations subnet to the Finance subnet resulted in "Destination Host Unreachable," proving the DENY rule was active.

- Infrastructure Health: show ip route confirmed all subnets remained "Connected" even with the filters active, ensuring the ACL did not break the underlying routing